# Networking & Communication Level 1 — Network Foundations

Topics Covered:

OSI Model - TCP/IP - IPv4 - IPv6 - Subnetting - Routing - Switching - NAT - VLANs - DNS Fundamentals - DHCP Fundamentals

## 1. OSI Model

The OSI (Open Systems Interconnection) model is a 7-layer framework that explains how data moves across a network. It is primarily a learning and troubleshooting model, not the exact design used by every real-world protocol.

## The 7 Layers of OSI:

- Physical Layer (Layer 1)

  - Sends raw bits over cables, fiber optics, or radio signals

  - Deals with voltage levels, pin layouts, and transmission media

  - Data unit: bits

- Data Link Layer (Layer 2)

  - Handles local delivery using MAC addresses

  - Creates and checks frames

  - Detects errors in frames

  - Data unit: frames

- Network Layer (Layer 3)

  - Routes packets between different networks using IP addresses

  - Determines the best path for data

  - Data unit: packets

- Transport Layer (Layer 4)

  - Provides end-to-end delivery using TCP (reliable) or UDP (fast but unreliable)

  - Handles segmentation, reassembly, and error correction

  - Data unit: segments (TCP) or datagrams (UDP)

- Session Layer (Layer 5)

  - Manages dialogs and sessions between applications

  - Controls when communication starts, continues, and ends

- Presentation Layer (Layer 6)

  - Handles data formatting, encryption, and compression

  - Ensures data is in a format the application can use

- Application Layer (Layer 7)

  - Provides network services to user applications

  - Includes protocols like HTTP, DNS, SMTP, FTP, and SSH

  - This is where users interact with the network

Simple Memory Tip:

Lower layers move data, middle layers direct and deliver it, and upper layers let applications use the network.

## 2. TCP/IP Model

The TCP/IP model is the practical model used by the Internet and most modern networks. It is simpler than OSI and usually described in 4 layers:

| TCP/IP Layer | Corresponding OSI Layers | Main Function |
| --- | --- | --- |
| Link Layer | Physical + Data Link | Hardware and local network delivery |
| Internet Layer | Network | IP addressing and routing |
| Transport Layer | Transport | TCP or UDP communication |
| Application Layer | Session + Presentation + Application | Protocols like HTTP, DNS, SMTP, FTP |

- Link Layer: Covers hardware (cables, Wi-Fi) and local delivery using MAC addresses

- Internet Layer: Handles IP addresses and routing (IPv4/IPv6)

- Transport Layer: Uses TCP (reliable) or UDP (fast) for communication between devices

- Application Layer: Includes all user-facing protocols (HTTP, DNS, SMTP, FTP, SSH)

TCP/IP matches how devices actually communicate on the Internet, while OSI is more useful for understanding concepts.

## 3. IPv4

IPv4 (Internet Protocol version 4) is the older and most widely known IP addressing system.

## Key Characteristics:

- Uses 32-bit addresses → about 4.3 billion possible addresses

- Written in dotted decimal format: 192.168.1.10

- Four octets, each ranging from 0 to 255

## Structure:

- Split into network portion and host portion

- Network part: identifies the network

- Host part: identifies a specific device on that network

## Private IPv4 Ranges (not routable on the Internet):

- 10.0.0.0 to 10.255.255.255 (10.0.0.0/8)

- 172.16.0.0 to 172.31.255.255 (172.16.0.0/12)

- 192.168.0.0 to 192.168.255.255 (192.168.0.0/16)

## Common IPv4 Terms:

- Subnet mask: Defines network vs. host bits

- Default gateway: Router for outside-network traffic

- Broadcast address: Sent to all devices in a subnet

- CIDR notation: /24, /16, etc. (e.g., 192.168.1.0/24)

IPv4 addresses are limited, so NAT is widely used to allow many private devices to share one public address.

## 4. IPv6

IPv6 (Internet Protocol version 6) is the newer version designed to solve IPv4 exhaustion and improve modern networking.

## Key Characteristics:

- Uses 128-bit addresses → enormous number of unique addresses

- Written in hexadecimal with colons: 2001:db8:abcd:0012::1

- Leading zeros can be removed

- One continuous run of zero groups can be compressed with :: (once per address)

## Advantages over IPv4:

- No address exhaustion

- Easier auto-configuration

- Better multicast support

- Less need for NAT

- Simplified routing and addressing

## IPv6 Address Types:

- Global unicast: Public Internet addresses

- Link-local: Used within a single network segment (starts with fe80::)

- Unique local: Private network addresses (starts with fc00::/fd00::)

- Multicast: Sent to multiple destinations

- Loopback: ::1 (like IPv4's 127.0.0.1)

IPv6 does not use broadcast. Instead, it uses multicast and anycast. Many networks run dual-stack (both IPv4 and IPv6).

## 5. Subnetting

Subnetting divides one larger network into smaller networks to improve organization, security, and traffic management.

## Key Concepts:

- Subnet mask / prefix length: Tells how many bits are for the network

  - Example: 255.255.255.0 = /24 (24 bits for network)

- Network address: First address in a subnet (not usable)

- Broadcast address: Last address in a subnet (not usable)

- Usable host range: Addresses between network and broadcast

## Host Calculation (IPv4):

Usable hosts=2host bits−2

Usable hosts=2

host bits

−2

Example: /24 network

- Total addresses: 256

- Usable hosts: 254 (256 - 2)

## Benefits of Subnetting:

- Reduces broadcast traffic

- Separates departments/systems

- Improves security and management

## 6. Routing

Routing moves packets between different networks using routers.

## How Routing Works:

- Router checks its routing table

- Decides the next hop based on destination IP

- Forwards packet toward the destination network

## Types of Routing:

| Type | Description |
| --- | --- |
| Static | Manually configured; good for small/simple networks |
| Dynamic | Uses protocols like RIP, OSPF, BGP to learn paths automatically |

## Key Points:

- Routing separates broadcast domains

- Connects multiple IP networks

- Default gateway: Router a device uses for outside-network traffic

- Essential for Internet communication

## 7. Switching

Switching forwards frames inside a local network (Layer 2 of OSI).

## How Switches Work:

- Build a MAC address table by observing source MAC addresses

- When a frame arrives, look up destination MAC and forward to the correct port

- If MAC is unknown, flood the frame to other ports in the same VLAN

## Benefits over Hubs:

- Reduces unnecessary traffic

- Improves network performance

- Core of modern LAN design

## 8. NAT (Network Address Translation)

NAT changes IP address information as traffic passes through a router/firewall.

## Main Purpose:

- Connect private networks to the public Internet

- Conserve IPv4 addresses

## Common NAT Types:

| Type | Description |
| --- | --- |
| Static NAT | One private → one public address |
| Dynamic NAT | Uses a pool of public addresses |
| PAT (NAT Overload) | Many private hosts → one public address (using ports) |

- Adds a layer of hiding between internal and external networks

- Not a full security solution but makes direct inbound access harder

## 9. VLANs (Virtual LANs)

A VLAN divides one physical switch network into multiple logical networks.

## Benefits:

- Improves security

- Reduces broadcast traffic

- Makes network management more flexible

## Example:

Place HR, Finance, and Guest devices into different VLANs on the same switch.

## Port Types:

- Access port: Carries traffic for one VLAN

- Trunk port: Carries traffic for multiple VLANs (uses IEEE 802.1Q tagging)

## Communication Between VLANs:

Requires routing (router or Layer 3 switch)

## 10. DNS Fundamentals

DNS (Domain Name System) translates human-friendly names (e.g., example.com) into IP addresses.

## DNS Process:

- Device asks DNS resolver for IP address

- Resolver may query:

  - Root servers

  - TLD servers (.com, .in, etc.)

  - Authoritative servers for the domain

## Common DNS Record Types:

| Type | Purpose |
| --- | --- |
| A | IPv4 address |
| AAAA | IPv6 address |
| CNAME | Alias for another name |
| MX | Mail exchange server |
| NS | Name server |
| TXT | Text data (e.g., verification) |

- DNS uses caching for faster repeated lookups

- DNS problems can cause websites to fail even when servers work

## 11. DHCP Fundamentals

DHCP (Dynamic Host Configuration Protocol) automatically gives devices network settings.

## What DHCP Provides:

- IP address

- Subnet mask

- Default gateway

- DNS server

## DORA Process:

- Discover: Client finds a DHCP server

- Offer: Server offers settings

- Request: Client requests the offered settings

- Acknowledge: Server confirms the lease

## Key Points:

- Reduces manual work

- Prevents address conflicts

- Lease is temporary → client must renew before expiration

- Can reserve specific addresses for particular devices

## 12. Putting It Together

Example Network Flow:

- Laptop joins Wi-Fi

- Gets IP from DHCP

- Asks DNS for website name

- Sends traffic through a switch

- Reaches Internet through a router using NAT

- VLANs separate different device groups

- Subnetting divides address ranges

- Routing connects those ranges

- TCP/IP explains how traffic is packaged and delivered

Understanding these topics together is more useful because real networks use all of them simultaneously.

## Quick Glossary

- IP address: Logical address identifying a device on a network

- MAC address: Hardware address used on the local network

- Packet: Data unit at the network layer

- Frame: Data unit at the data link layer

- Router: Device connecting different networks

- Switch: Device connecting devices within a LAN

- Broadcast: Traffic sent to all devices in a segment

- Lease: Temporary DHCP assignment of network settings